| 6.pielikums “Project Specific Requirements” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Control points | Design BIM stage |  | Construction stahe (BIM renewal, BIM improvement) 3 |  |  | As-built model 3 | Asset Information model 3 |  |
|  | Due date1 | - |  | On going |  |  | Until the comissioning | Until handovered to the Client |  |
| Deliverables: 3D BIM models |  |  |  |  |  |  |  |  |  |
|  | 3D BIM model/Detailing levels: |  |  |  |  |  |  |  |  |
|  | AR (IN, IE, BAA)* | LOD 350 |  | LOD 350 |  |  | LOD 350 | LOD 350 |  |
|  | GP, TS-L | LOD 300 |  | LOD 300 |  |  | LOD 300 | LOD 300 |  |
|  | BK | LOD 300 |  | LOD 300 |  |  | LOD 300 | LOD 300 |  |
|  | AVK (AVK-A, AVK-V, AVK-K)* | LOD 300 |  | LOD 300 |  |  | LOD 300 | LOD 300 |  |
|  | UK (UK-U, UK-K)*, UKT, LKT | LOD 300 |  | LOD 300 |  |  | LOD 300 | LOD 300 |  |
|  | EL, ELT | LOD 300 |  | LOD 300 |  |  | LOD 300 | LOD 300 |  |
|  | ESS (ESS-PK, ESS-AS, ESS-TK, ESS-VAS,ESS-VN)*, EST | LOD 300 |  | LOD 300 |  |  | LOD 300 | LOD 300 |  |
|  | SM | LOD 300 |  | LOD 300 |  |  | LOD 300 | LOD 300 |  |
|  | UATS | LOD 300 |  | LOD 300 |  |  | LOD 300 | LOD 300 |  |
|  | BHS | LOD 300 |  | LOD 300 |  |  | LOD 300 | LOD 300 |  |
|  | Wayfinding | LOD 100 |  | LOD 100 |  |  | LOD 350 | LOD 350 |  |
|  | Federated model |  |  |  |  |  |  |  |  |
|  | … |  |  |  |  |  |  |  |  |
| Other BIM deliverables |  |  |  |  |  |  |  |  |  |
|  | Photogrammetry |  |  |  |  |  |  |  |  |
|  | Laserscanning |  |  |  |  |  |  |  |  |
|  | Laser scanning of the existing situation before the design or construction phase |  |  |  |  |  |  |  |  |
|  | 4D: simulation of the construction process |  |  |  |  |  |  |  |  |
|  | 4D: logistics, waste management, intensity |  |  |  |  |  |  |  |  |
|  | 4D: renewal and status management |  |  |  |  |  |  |  |  |
|  | 4D: Constructability Analysis |  |  |  |  |  |  |  |  |
|  | Analysis of environmental accessbility |  |  |  |  |  |  |  |  |
|  | Acoustic analysis |  |  |  |  |  |  |  |  |
|  | Analysis of people flow and evacuation options |  |  |  |  |  |  |  |  |
|  | Fire simulation |  |  |  |  |  |  |  |  |
|  | Lighting simulation |  |  |  |  |  |  |  |  |
|  | Security analysis |  |  |  |  |  |  |  |  |
|  | Location analysis |  |  |  |  |  |  |  |  |
|  | Analysis of insolation |  |  |  |  |  |  |  |  |
|  | Construction site risk analysis |  |  |  |  |  |  |  |  |
|  | Energy efficiency analysis |  |  |  |  |  |  |  |  |
|  | Sustainability analysis |  |  |  |  |  |  |  |  |
|  | 5D (Execution corresponds to the analysis of the situation) |  |  |  |  |  |  |  |  |
|  | Visualization |  |  |  |  |  |  |  |  |
|  | Virtual reality |  |  |  |  |  |  |  |  |
|  | Augmented reality |  |  |  |  |  |  |  |  |
|  | Cost management |  |  |  |  |  |  |  |  |
|  | Model models for management |  |  |  |  |  |  |  |  |
|  | … |  |  |  |  |  |  |  |  |
|  | 1 the design phase is determined in months from the moment of signing the contract, for each project individually |  |  |  | Information requirements for the project: |  |  |  |  |
|  | 2 Sections of the construction project indicated in brackets may be divided into separate models |  |  |  |  |  |  |  |  |
|  | 3 Updating and supplementing information under author supervision |  |  |  |  |  |  |  |  |
|  | BIM delivery method | Projektē un Būvē / projekts-iepirkums-būvdarbi / projektēšana |  |  | Data group |  |  | Yes/No |  |
|  |  |  |  |  | AR, Windows and doors |  |  | Yes 4 |  |
|  | Type of provision of CDE | Dalīta cikla VDV / Pilna cikla VDV |  |  | AR, Finishes |  |  | Yes 4 |  |
|  |  |  |  |  | AR, Rooms |  |  | Yes |  |
|  |  |  |  |  | AR, Building elements |  |  | yes 4 |  |
|  |  |  |  |  | BK, Structurel elements |  |  | Yes 4 |  |
|  | Responsible for CDE provision (in case of full-cycle VDV) | Piegādātājs / Pasūtītājs / N/A |  |  | EL, VS, ESS, Cable trays |  |  | Yes |  |
|  |  |  |  |  | Engineering networks |  |  | Yes |  |
|  |  |  |  |  | Asset Information |  |  | Yes |  |
|  |  |  |  |  | 4 A smaller amount of information is possible for the elements to be reconstructed and restored, upon prior agreement with the customer. |  |  |  |  |
|  | Responsible for providing laser scanning | Piegādātājs / Pasūtītājs / N/A |  |  | Technical parameters (if laser scanning is provided by the Customer) |  |  |  |  |
|  |  |  |  |  | Laser scanner parameters | Distance accuracy error | N/A |  |  |
|  |  |  |  |  |  | 3D point accuracy | N/A |  |  |
|  |  |  |  |  | Point cloud parameters | Registration deviations | N/A |  |  |
|  |  |  |  |  |  | Point cloud density | N/A |  |  |

In addition to the special requirements-

- For BK model deliverables, only steel structures should be provided with LOD 400 level (in construction and As-build model);

- *LOD 400 Reinforcement modeling for a reinforced concrete structure, the Supplier can give a price offer for the creation of individual reinforcement models;

- There is no need to model indoor electrical and low current cables and wires, as well as external engineering networks;

- The list of exceptions can be supplemented during the implementation of the project, upon agreement between the customer and the supplier, as well as by describing each specific exception in the Post-contract BIM execution plan.

- Create a BIM model (all sections of the construction project) for the cooling building (area - 222m²) at the new terminal of the 6th round;

- Provide BIM models for the new engineering communications in the existing terminal;

- About the Wayfinding model – the graphic level must be such that you can visually read/see in the 3D model what the signs indicate. (Text, directions, symbols, etc.)