2.pielikums

Atklāts konkurss ar identifikācijas Nr. PRO-2023/

Annex No. 2

Terms of Reference of Open Tender (ID Nr. PRO-2023/)

TEHNISKĀ SPECIFIKĀCIJA un TEHNISKĀ PIEDĀVĀJUMA FORMA/ TECHNICAL SPECIFICATION and TECHNICAL TENDER FORM

Atklāts konkurss “Inčukalna PGK ģeofizikālais aprīkojums”, ID Nr. PRO-2023/

Open tender “Geophysical equipment for Incukalns UGS”, ID Nr. PRO-2023/

TECHNICAL REQUIREMENTS

Geophysical equipment for Incukalns Underground Gas Storage

1. Working environment data

1.1Environmental data:

      - Ambient temperature: -40 C to +35C;

      - Average air humidity: 89%;

      - Height above sea level: 70m.

1.2.Working environment: Natural gas, CH4 (density 0,688g/m3; working temperature: -10 0C to +30 0C and well mud/killing liquid density up to 1,53g/cm3). Formation water with mineralisation up to 110 g/l.

1.3. Debris and additional properties:

| Debris in gas (sand, dust, etc.) | up to 25 g/m3 |
| --- | --- |
| Water factor | up to 0,25 g/m3 |
| Water mineralisation | up to 110 g/m3 |

1.4.Natural gas components:

| Components | Composition, % | Components | Composition, % |
| --- | --- | --- | --- |
| Methane (CH4) | 96,38% | Hexane | 0,001% |
| Ethane (C2H6) | 2,18% | CO2 | 0,13% |
| Prophane (C3H8) | 0,44% | Nitrogen | 0,71% |
| Izobuthane (i-C4H10) | 0,075% | Oxygen | 0,01% |
| Buthane (n-C4H10) | 0,07% | H2S | 0,0189 mg/m3 |
| Izopenthane (C5H12) | 0,017% | Ethilmercapthane | 0,0302 g/m3 |
| Penthane (n-C5H12) | 0,01% | Overall H2S | 0,1388 mg/m3 |

Ģeofizikālais aprīkojums/Geophysical equipment

| - Required Geophysical equipments technical parameters: |  |  | PROVIDED BY APPLICANT, TECHNICAL TENDER* |
| --- | --- | --- | --- |
| Pulsed neutron-neutron logging tool for gas saturation measurements |  |  |  |
| Working pressure rating | 15 000 | psi |  |
| Working temperature range | – 40 to + 35 | 0С |  |
| Tool length | Up to 85” | inch |  |
| Tool diameter | Up to 2” | inch |  |
| Power | Up to 150VDC/100mA | voltage |  |
| Desired weight | Up to 20kg | kg |  |
| Operations | Distance turn on/off |  |  |
| Sinker bar | Lead or stainless steel, up to 30kg, up to 2,50m. | Length (m), weight (kg) |  |
| Output signals, connection | Min. 3 channels/sensors (Gamma ray, borehole temperature, coupling-metal location), digital, compatible with provided encoder. Appropriate cable head (for electric wireline with a diameter 5/16”, 8mm LN, up to 0.50” string) with voltage sensor, up to 2A. Max tension up to 200kN. Desired an appropriate pin thread. | Connection type |  |
| Pulsed neutron-neutron source, type | Neutron generator, He3 source or other, with energy emitted up to 15 MeV, but not less than 0.020eV. The output neutron fluxes up to 2 x 108 or equivalent. Pulse duration – 1 to 3 µs, 60-80ms between pulses | Emitted energy eV-Mv |  |
| Pulsed neutron-neutron (PNN) gamma ray sonde (logging tool) for gas saturation measurements is meant for natural gas saturation, behind the casing fluid and porosity determination. Also, it ensures borehole temperature logs. The tools should ensure data from both enclosed and open hole wells. |  |  |  |
| Data encoder |  |  |  |
| Connection type | Rotor, compatible with the PNN tool impulse count range and connection cable signals and registrator |  |  |
| RPM count rate min-max | 6000-12000 min-1 |  |  |
| Material type | Stainless steel |  |  |
| Working temperature | – 40 to + 35 | 0С |  |
| Rotor torque | Up to 6 x 10-6 kgm2 (0,01-0,05 Nm) |  |  |
| Power | 5V (+/-5%) 10-30VDC, 55-150 Ma, 300kHz, CE conformity, EN-50082-2; NE 50081-2; NE 55011 B class |  |  |
| Well logging data acquisition system/registrator |  |  |  |
| Compatibility | Compatible to visualize the signals from the used PNN tool and encoder to an graphical interface. Used with MS Windows (up to date versions) | Provide description/manual documentation about the logging system |  |
| Information output, storage | The signals from the tool and data acquisition shall be monitored and shown real-time. The data shall be saved on the hardware. The hardware (PC - portable) with the appropriate software shall be offered by the tenderer | Provide description/manual documentation about the logging system |  |
| Data acquisition modules | The registrator shall include: Single/Dual cement bond module; Gamma ray module, Neutron (single/dual) detection module; Pulsed neutron module; temperature, noise, Casing inspection modules, etc., which are included in the offered registrator | Provide description/manual documentation about the logging system |  |

3. Amount of deliverables

1 (one) pulsed neutron-neutron logging tool according to the technical specification and necessary spare parts, additional components (communication section, neutron detector section, neutron generator). 1 (one) Data and rotation Encoder with necessary spare parts, components. 1 (one) compatible logging data acquisition system with a portable PC and software according to the described technical parameters;

4. Technical documentation and guarantee time

The Contender provides the quality compliance sertificates of the delivered equipment, technical documentation of the equipment including manuals and maintenance and work instructions of the equipment in latvian or english language. The producer provides a guarantee not less than  1 (one) year after the delivery. The Contender provides training to the personell of Incukalns UGS.

5. Delivery place and time

The equipment shall be delivered to JSC “Conexus Baltic Grid” Inčukalna pazemes gāzes krātuve, Krimuldas pagasts, Siguldas novads, Latvija, LV-2144 (DDP Incoterms). Delivery time – until 30.03.2024.

The equipment shall be packed accordingly to prevent any damage during its transportation.The packaging musto contain the following information:

- Full name and information of the equipments producer;

- Full name of the equipment, content;

- Bruto weight, neto weight, packaging dimensions

* Tehniskajā piedāvājumā norādīt precīzus tehniskos datus atbilstoši tehniskajos parametros norādītajam, piedāvātās Preces ražotāju un modeli, kā arī citu informāciju, ko Pretendents uzskata par nepieciešamu norādīt / Indicate in the technical tender the exact technical data in accordance with the technical parameters, the manufacturer and model of the offered Product, as well as other information that the Tenderer deems necessary to indicate.

Paraksts/signature: ________________________

Parakstītāja vārds, uzvārds un amats/ Given name, surname and position: _________________

Datums/date: _______________